VLSI System Design (Graduate Level)

Fall 2018

HOMEWORK III

REPORT

Must do self-checking before submission:

Compress all files described in the problem into one tar

All SystemVerilog files can be compiled under SoC Lab environment

All port declarations comply with I/O port specifications

Organize files according to File Hierarchy Requirement

No any waveform files in deliverables

Student name:	_____杜冠勳___________

Student ID:	_____N26094883 ___________

| Cache Architecture |  |
| --- | --- |
| FSM(READ) |  |
| Figure 1 |  |
| STATE EXPLANATION |  |
| IDLE | Waiting state ,if there is any cpu read signal ,the next state will be check hit read |
| CHECK_HIT_READ | Check if the search in cache is hit or not the hit condition will be (TA_out==TA_in )&&(single_valid_data) if miss the next condition will be send read mem1 if hit the next condition will be IDLE |
| SEND_READ_MEM1~4 | The state will send the request to cpu wrapper to get data from main memory and the next state will be read_mem |
| READ_MEM1~4 | The state is waiting for the finsh of reading data from main memory after reading a cache line data(128bits) reading 4 times from main memory the next state will return to IDLE state |
| Detailed | Because when the design of CPU_wrapper receive the request from cache ,it will active D_wait at the same time thus it chould have two state ,one for sending the request signal One for waiting the D_wait signal to drop. More detailed is explan by waveform |
| Special design | Whether the cpu hit or not the cache read the data array by index however only hit condition will return the data from data array. In normal case the controller should know the hit condition first and send the read signal to dataarray In my case ,sending read signal to tag array for checking hit condition and getting data from data array are happen at the same time. |

| Cache Architecture |  |
| --- | --- |
| FSM(READ) |  |
| Figure 2 |  |
| STATE EXPLANATION |  |
| IDLE | Waiting state ,if there is any cpu read signal ,the next state will be check hit write |
| CHECK_HIT_WRITE | Check if the search in cache is hit or not . the hit condition will be (TA_out==TA_in )&&(single_valid_data). if hit the next condition will be WRITE_HIT and send the wrinting request to CPU_wrapper rat the same time. if hit the next condition will be WRITE_MISS and send the wrinting request to CPU_wrapperat the same time. |
| WRITE_HIT | Write through to the memory and cache Check if writing operation to main memory is finish or not . If finish it will return to IDLE |
| WRITE_MISS | Write through to the memory Check if writing operation to main memory is finish or not . If finish it will return to IDLE |
| Detailed | Because when the design of CPU_wrapper receive the request from cache ,it will active D_wait at the same time thus it chould have two state ,one for sending the request signal One for waiting the D_wait signal to drop. More detailed is explan by waveform |

| Waveform |  |
| --- | --- |
| Cache |  |
| Figure 3 |  |
| Explanation |  |
| When the cpu send req to cache the core wait active(green block) |  |
| READ_miss | (red block) IM read instruction When the cpu start the cache has no data thus write miss happen When the write miss happen ,cache send request to cpu wrapper it can explan by wrapper ARADDR get the address sent by cpu 0 4 8 c The wrapper send the data to cache if(4'd0<=offset&&offset<4'd4) it will return the first data to cpu that get from CPU_wrapper which is this case if(4'd4<=offset&&offset<4'd8) it will return the second data that get from CPU wrapper to cpu if(4'd8<=offset&&offset<4'd12) it will return the third data that get from CPU wrapper to cpu else it will return the forth data that get from CPU wrapper to cpu |
| READ_hit | (yellow block) Because there is a write miss at reading address=0 so The cache will update one cache line which address=0 4 8 C strored in cache Therefore when reading address = 0 4 8 hit operation should be hit in cache which can proved by the core wait time is obviously shorten than privous. |

| Waveform |  |
| --- | --- |
| Cache |  |
| Figure 4 |  |
| Explanation |  |
| When the cpu send req to cache the core wait active(green block) |  |
| WRITE_miss (Figure 5) | (red block) 74C is the store instruction :SW the third stage counted by cpu_stall sending the write requset to cache controller When the cpu send the write signal ,the FSM state:STATE_CHECK_HIT_WRITE(8) observe that if the hit hit condition happened or not When the write miss happen:state(9),the controller will not send the write signal to data array, valid array and tag array (PINK BLOCK) but it send the write signal to CPU WRAPPER (YELLOW BLOCK) The State will wait the D_wait signal sent by CPU wrapper back to zero and return to IDLE STATE(0) (red block) The cpu_wait signal is still active high because instruction cache is reading data from memory |

| Waveform |  |
| --- | --- |
| Cache |  |
| Figure 6 |  |
| Explanation |  |
| When the cpu send req to cache the core wait active(green block) |  |
| WRITE_hit | (red block) 7fc is the store instruction :SW the third stage counted by cpu_stall sending the write requset to cache controller When the cpu send the write signal ,the FSM state:STATE_CHECK_HIT_WRITE(8) observe that if the hit hit condition happened or not When the write hit happen:state(10),the controller will send the write signal to data array(PINK BLOCK) The reson why web data is f3ff is because it’s SH instruction Web data is also contrill by offset data The write hot operation also send the write signal to CPU WRAPPER (YELLOW BLOCK) The State will wait the D_wait signal sent by CPU wrapper back to zero and return to IDLE STATE(0) (red block) The cpu_wait signal is still active high because instruction cache is reading data from memory |

| Waveform |  |  |
| --- | --- | --- |
| Instruction |  | LH |
| ff041383 |  | lh t2,-16(s0) |
| Figure 7 Figure 8 |  |  |
| Explanation |  |  |
| ID | The ID stage decode opcode and decode read mem signal The mem_out_lowis active and the mem_out_half ,padding zero is not active because it ‘s the load half word(signed extended) instruction(red block) |  |
| EXE | The EXE stage calculate the address that is going to read from memory(green block) |  |
| MEM | READ data from cache because the data is hit in cache(blue block) Read data is core_out=ccccccc |  |
| WB | Because the lh operation and the address[1]=0 thus the write back data shold be ffffcccc(yellow part) |  |

| Waveform |  |  |
| --- | --- | --- |
| Instruction |  | LBU |
| ff044e03 |  | lbu t3,-16(s0) |
| Figure 9 Figure 10 |  |  |
| Explanation |  |  |
| ID | The ID stage decode opcode and decode read mem signal The mem_out_half padding zero is active and the mem_out_low is not active because it ‘s the load byte word (unsigned signed extended)instruction(red block) |  |
| EXE | The EXE stage calculate the address that is going to read from memory(green block) |  |
| MEM | READ data from cache because the data is hit in cache(blue block)Read data is core_out=ccccccc |  |
| WB | Because the lh operation and the address[1:0]=00 thus the write back data shold be 000000cc(yellow part) |  |

| Waveform |  |  |
| --- | --- | --- |
| Instruction |  | LHU |
| ffe41723 sh t5,-18(s0) |  | lhu t4,-16(s0) |
| Figure 11 Figure 12 |  |  |
| Explanation |  |  |
| ID | The ID stage decode opcode and decode read mem signal The mem_out_half padding zero is active and the mem_out_low is not active because it ‘s the load half word (unsigned signed extended)instruction(red block) |  |
| EXE | The EXE stage calculate the address that is going to read from memory(green block) |  |
| MEM | READ data from cache because the data is hit in cache(blue block)Read data is core_out=ccccccc |  |
| WB | Because the lh operation and the address[1]=0 thus the write back data shold be 0000cccc(yellow part) |  |

| Waveform |  |  |
| --- | --- | --- |
| Instruction |  | SH |
| ffe41723 |  | sh t5,-18(s0) |
| Figure 13 Figure 14 |  |  |
| Explanation |  |  |
| ID | The ID stage decode opcode and decode read mem signal The mem_in_half is active (yellow block) |  |
| EXE | NONE |  |
| MEM | CPU send the write signal to cache to write data to memory . In this case the cache hit condition happened which can proved by the FSM state 0 >8>a(blue block) Data array.WEB is f3ff is controlled by address[1] and core type=5 sent by CPU (green block) MEMORY.WEB if 3 is controlled by address[1] and core type=5 sent by cache (red block) Web data :4’h3=4’b0011 which can proved SH store half word |  |
| WB | NONE |  |

| Simulation |  |
| --- | --- |
| rtl0 | rtl1 |
| rtl2 | Syn0 |
| Syn1 | Syn2 |

JASPERGOLD

Explanation

In my case when the core send write req to cache it will send send D_req for only one cycle . The reson why I only keep D_req for only one cycle is due to my CPU wrapper design. If the read signal(D_req) is always is keep active during write or read operation ,the CPU wrapper will lock in cycle for reading or writing . The lock condition is due to my CPU_wrapper 0 STATE.In 0 STATE The CPU_wrapper receive request will active D_wait at the same time because it’s combinaional circuit. Therefore in my cache design ,I have to keep my request for only 1 cycle preventing from recursive condition. Nevertheless ,the jaspergold verification has the assume that D_req will remain high during D_wait which will not cover my design.

| Synthesize result(8ns) |  |
| --- | --- |
| TIMING |  |

| AREA |  |
| --- | --- |
| POWER |  |

| APR |  |
| --- | --- |
| Before Placement Verify Geometric |  |
| Before Placement Verify Connectivity |  |
| Post_route Setup_verify |  |
| Post_route holdup_verify |  |

| Post_cts Setup |  |
| --- | --- |
| Post_cts Holdup |  |
| ANTENNA & GEOMETRIC CONNECTIVITY & VERIFY |  |

Floorplan_View

Amoeba_View

Physical_view

| Superlint |  |
| --- | --- |
| Warning Explantion | File format 為命名沒有和module 同名 不影響合成 Coding style 是因為寫法習慣的問題 combination 電路中 會有接線來自 sequential 的區塊 不影響合成 一樣是合出flipflop |
| Superlint within 95% | 1-(49+23)/9923 = 99.27% >95% |

心得

我從這次作業中學習到cache 的設計,其中有諸多考量是當時學習祭祖時不會想到的,像是 在更新cache line 時 因為 128 bits 在 hits 要回傳資料時 不該用 offset 為 0 4 8 c 的方式判斷要回傳哪筆資料 而是應該用0~3 4~7 8~b c~f 的 方式去做判斷 因為 address 有可能不是4的倍數