VLSI System Design (Graduate Level)

Fall 2018

HOMEWORK III

REPORT

Must do self-checking before submission:

Compress all files described in the problem into one tar

All SystemVerilog files can be compiled under SoC Lab environment

All port declarations comply with I/O port specifications

Organize files according to File Hierarchy Requirement

No any waveform files in deliverables

Student name:	_____杜冠勳___________

Student ID:	_____N26094883 ___________

| Cache Architecture |  |
| --- | --- |
| FSM(READ) |  |
| STATE EXPLANATION |  |
| IDLE | Waiting state ,if there is any cpu read signal ,the next state will be check hit read |
| CHECK_HIT_READ | Check if the search in cache is hit or not the hit condition will be (TA_out==TA_in )&&(single_valid_data) if miss the next condition will be send read mem1 if hit the next condition will be IDLE |
| SEND_READ_MEM1~4 | The state will send the request to cpu wrapper to get data from main memory and the next state will be read_mem |
| READ_MEM1~4 | The state is waiting for the finsh of reading data from main memory after reading a cache line data(128bits) reading 4 times from main memory the next state will return to IDLE state |
| Detailed | Because when the design of CPU_wrapper receive the request from cache ,it will active D_wait at the same time thus it chould have two state ,one for sending the request signal One for waiting the D_wait signal to drop. |

| Cache Architecture |  |
| --- | --- |
| FSM(READ) |  |
| STATE EXPLANATION |  |
| IDLE | Waiting state ,if there is any cpu read signal ,the next state will be check hit write |
| CHECK_HIT_WRITE | Check if the search in cache is hit or not . the hit condition will be (TA_out==TA_in )&&(single_valid_data). if hit the next condition will be WRITE_HIT and send the wrinting request to CPU_wrapper rat the same time. if hit the next condition will be WRITE_MISS and send the wrinting request to CPU_wrapperat the same time. |
| WRITE_HIT | Check if writing operation to main memory is finish or not . If finish it will return to IDLE |
| WRITE_MISS | Check if writing operation to main memory is finish or not . If finish it will return to IDLE |
| Detailed | Because when the design of CPU_wrapper receive the request from cache ,it will active D_wait at the same time thus it chould have two state ,one for sending the request signal One for waiting the D_wait signal to drop. |

Waveform

| Synthesize result(8ns) |  |
| --- | --- |
| TIMING |  |

| AREA |  |
| --- | --- |
| POWER |  |

| APR |  |
| --- | --- |
| Before Placement Verify Geometric |  |
| Before Placement Verify Connectivity |  |
| Post_route Setup_verify |  |
| Post_route holdup_verify |  |

| Post_cts Setup |  |
| --- | --- |
| Post_cts Holdup |  |
| ANTENNA & GEOMETRIC CONNECTIVITY & VERIFY |  |

Floorplan_View

Amoeba_View

Physical_view

| Simulation |  |
| --- | --- |
|  | Syn0 |
| Syn1 | Syn2 |

| Superlint |  |
| --- | --- |
| Warning Explantion | File format 為命名沒有和module 同名 不影響合成 Coding style 是因為寫法習慣的問題 combination 電路中 會有接線來自 sequential 的區塊 不影響合成 一樣適合出flipflop |
| Superlint within 95% | / = >95% |

心得